Configuration of Zyxel Line Bonding Modem  ( Model- P-663HN-51 )

Step No. 1]   Open router page

Http://192.168.1.1

Username:- admin

Password :- 1234

You will get following screen

Step No. 2] Click on Advance Setup

You will get following screen

Tick mark under Remove  and click on Remove button to remove default WAN interface setting.

And you will get screen as below-

Step No. 3] Creating and configuration of WAN interface

Click on ADD button

You will get screen as below-

Change the value of VCI

VPI 0 and VCI 32  and click on Next button

You will get screen as below-

Select PPP over Ethernet (PPPoE), Change MTU value as 1492 and click on Next button

You will get screen as below-

Enter     Username as Telephone no. with suffix @a

Password as Customer’s CA no.

Tick mark on Enable NAT

Click on Next button

You will get screen as below-

No change at this stage simply Click on Next button

You will get screen as below-

Click on Save button.

You will get screen as below-

Click on Save and Reboot button

Now router is rebooting please wait

Step no. 4] Wireless setting

Click on Wireless

You will get screen as below-

Click on Security

You will get screen as below-

Select  Network Authentication: as WPA2-PSK

WPA pre-Shared Key: [ password must be minimum 8 ASCII character e.g. asedf!@6]

WPS Encription: as TKIP/AES

Click on Save/Apply button

StepNo. 5] Save and Reboot configuration

Click on Management

Click on Save/Reboot

Click on Save/Reboot button

Now Router is rebooting. Wait for 2 minute.

Step No. 6] Restore Default Setting

Click on Management

Click on Setting

Click on restore Default Setting button

DE BB HD

MTNL MUMBAI

29-10-2016.